ARINC 615A Tool Suite — CLI

Installation, Operation and Test Procedure

| Item | Value |
| --- | --- |
| Document title | ARINC 615A CLI — Installation, Operation and Test Procedure |
| Applies to | arinc_615a_operation (CLI), all 16 commands |
| Repository | arinc-615a-tool-suite |
| Platforms | Windows x64 (MSVC 2022); Linux x64/arm64 (GCC 13+) |
| Document status | Issue 1 |
| Licence | MPL-2.0 — upstream project by Thomas Vogt |

Verification note

Every expected result in Section 9 marked VERIFIED was produced by executing the stated command on a Windows x64 machine and copying the actual output. Results marked NOT EXECUTED are specified from the tool's own option definitions but were not run, because they require either ARINC 615A target hardware on the network or a Linux environment. No expected output in this document is invented.

# 1  Purpose and scope

This document is a step-by-step procedure for installing, operating and testing the ARINC 615A command-line data loader. It is written to be followed literally: every step is a command to type, with the output you should see.

In scope:

Installing all prerequisites and dependencies on Windows and Linux.

Building the application from source.

Running each of the 16 commands.

A test procedure with pass/fail criteria.

Diagnosing the failures that are known to occur.

Out of scope:

The target-side implementation (the avionics box). This tool is the host.

Airworthiness, certification or approval of any data-loading activity.

The ARINC standards themselves, which are published by SAE ITC.

## 1.1  Acronyms

| Term | Meaning |
| --- | --- |
| DLP | Data Loader Protocol |
| FIND | Find Identification of Network Devices — the discovery protocol |
| IRQ / IAN | FIND Information Request / Information Answer |
| LRU | Line Replaceable Unit — an avionics box |
| MSM | Media Set Manager — the local ARINC 665 software store |
| THA | Target Hardware Application — the software on the target that speaks 615A |
| TFTP | Trivial File Transfer Protocol (RFC 1350), the transport |
| P/N | Part Number |

# 2  System overview

ARINC 615A is a file-driven protocol layered on TFTP. The host (this tool) and the target exchange files with fixed three-letter extensions; the state machine is driven by which file arrives next. FIND is the exception — a plain UDP request/answer pair used to discover targets before a transfer.

| Extension | File | Direction |
| --- | --- | --- |
| LCI | Load Configuration Initialisation | host to target |
| LCL | Load Configuration List (part numbers, THW IDs) | target to host |
| LCS | Load Configuration Status | target to host |
| LUI / LUR / LUS | Upload Initialisation / Request / Status | both |
| LNR / LNA / LNS | Download Request / Answer / Status | both |

Figure 1  —  System context: the tool is the host; the data path is files over TFTP

Figure 2  —  Six layers, one direction. Calls descend; results return through handlers

Two behaviours matter operationally. The exception timer (--dlp-timeout, default 13 s) fails an operation if the target stops sending status files, rather than hanging. And the first Ctrl-C is a graceful protocol abort, not a kill — it tells the target to stop so it is not left mid-load in an undefined state. A second Ctrl-C terminates hard.

Figure 3  —  Abort model: the first Ctrl-C is a protocol abort, the second is a hard stop

## 2.1  Where ARINC 615A sits in the protocol stack

ARINC 615A is an application-layer protocol. It defines no transport of its own: it rides on TFTP over UDP over IPv4 over Ethernet. Everything below the session layer is provided by the operating system and the network, which is why interfacing the loader to another system is mostly a networking exercise rather than a software one.

Figure 4  —  ARINC 615A mapped onto the OSI reference model, with the ports this build uses

## 2.2  Protocols in use while an operation runs

A packet capture taken during a load shows more than ARINC 615A. The table lists everything on the wire and whether this software drives it.

| Protocol | Layer | Role during an operation | Driven by |
| --- | --- | --- | --- |
| ARINC 615A DLP | Application | Protocol files LCI, LCL, LCS, LUI, LUR, LUS, LNR, LNA, LNS | This tool |
| FIND | Application | Discovery: IRQ opcode 1, IAN opcode 2 | This tool |
| ARINC 665 | Presentation | Load and media set format carried in the transfer | This tool |
| TFTP (RFC 1350) | Session | Moves every protocol file and every load file | tftp library |
| TFTP options (RFC 2347-2349) | Session | Block size, timeout and transfer size negotiation | This tool |
| UDP | Transport | Connectionless datagrams; no delivery guarantee | Operating system |
| IPv4 | Network | Addressing and routing | Operating system |
| ARP | Network | Resolves target IP to a MAC address before any UDP flows | Operating system |
| ICMP | Network | Diagnostics only; not part of the protocol | Operating system |
| Ethernet 802.3 | Data link | Framing; carries the FIND broadcast | NIC and switch |

## 2.3  Ports and timers used by this build

| Purpose | Default | Source constant | Override |
| --- | --- | --- | --- |
| Data load (TFTP) | UDP 59 | DefaultArinc615aTftpPort | --server-port |
| FIND discovery | UDP 1001 | Find::DefaultPort | --find-port |
| TFTP packet timeout | 2 s | DefaultArinc615aTftpTimeout | --tftp-timeout |
| TFTP retries | 1 | DefaultArinc615aTftpRetries | --dlp-retries |
| DLP exception timer | 13 s | DefaultArinc615aDlpTimeout | --dlp-timeout |
| DLP retries | 1 | DefaultArinc615aDlpRetries | --dlp-retries |

The data-load port is UDP 59, not the TFTP well-known port 69. ARINC 615A-1 states the port explicitly and this build takes its default from that constant. If a firewall between host and target permits only port 69, the load will not start.

Figure 5  —  What is exchanged during an operation, and the three integration points

## 2.4  Interfacing the loader to other systems

The loader can be driven as a process or as a library; both are supported.

As a process: invoke the CLI and read its exit code. --targets-list produces machine-readable discovery output a wrapper can parse and feed back in.

As a library: link lib/arinc_615a and implement the ...OperationHandler interfaces to receive progress, status and completion callbacks. The CLI is itself only a thin consumer of that API, so everything it does is available to your own application.

Software to be loaded is supplied as ARINC 665 media sets managed through the Media Set Manager directory; ARINC 649 supplies the shared check-value functions.

Network conditions that must hold:

Host and target must share a broadcast domain for FIND to discover anything. Routers block broadcast; across a router, address the target directly with --target-address.

UDP 59 and UDP 1001 must not be filtered between host and target.

On a multi-homed host set --local-tftp-address and --local-find-address explicitly, or the sockets may bind to the wrong interface.

Figure 6  —  Interfacing the loader to an aircraft or bench network

# 3  Prerequisites

| Requirement | Windows | Linux |
| --- | --- | --- |
| Operating system | Windows 10/11 x64 | x86_64 or aarch64 |
| Compiler | Visual Studio 2022 C++ build tools | GCC 13 or newer (C++23) |
| Build system | CMake 4.3+ and Ninja | CMake 4.3+ and Ninja |
| Source control | Git | Git |
| Libraries from | vcpkg (approx. 84 packages) | distro packages |
| Privileges | normal user | sudo, for package installation |
| Disk space | approx. 10 GB (vcpkg build trees) | approx. 2 GB |
| Network | required — see 3.1 | required — see 3.1 |

## 3.1  Network access required at build time

The CMake configure step clones five sibling projects with FetchContent. There is no vendored copy and no offline fallback: without access to git.thomas-vogt.de, configure cannot complete.

| Host | Needed for |
| --- | --- |
| git.thomas-vogt.de | helper, arinc_649, arinc_665, tftp, commands |
| github.com | vcpkg clone (Windows); CMake binary (Linux, if distro CMake is too old) |
| Package mirrors | distro packages (Linux) or vcpkg source downloads (Windows) |

## 3.2  Time expectations

The first Windows build is slow and this is normal, not a fault. vcpkg builds roughly 84 packages; libiconv alone runs its autotools configure twice and was measured at 2.3 hours on the reference machine. Subsequent builds are served from the vcpkg binary cache and take seconds to a few minutes. Linux is minutes, because it uses distro packages and never builds libiconv from source.

# 4  Installation — Windows

## Step 4.1  Install the C++ toolset

Run in an elevated PowerShell or Command Prompt:

winget install --id Microsoft.VisualStudio.2022.BuildTools -e --override "--quiet --add Microsoft.VisualStudio.Workload.VCTools --includeRecommended"

Expected: winget reports successful installation. Skip if Visual Studio 2022 with the C++ workload is already present.

## Step 4.2  Install CMake and Git

winget install --id Kitware.CMake -e winget install --id Git.Git -e

CMake must be 4.3 or newer. The CMake bundled inside Visual Studio is 3.31 and is NOT sufficient; the setup script detects this and searches for a qualifying installation.

## Step 4.3  Obtain the source

git clone <repository-url> arinc-615a-tool-suite cd arinc-615a-tool-suite

## Step 4.4  Install, build and run — one command

This is the entire installation. It installs dependencies, builds, and runs the tool:

build.bat

To build and immediately run a specific command, pass its arguments through:

build.bat -c Find build.bat debug -c Find build.bat --no-run

Expected output, abbreviated:

============================================================== ARINC 615A Tool Suite install dependencies -> build (release) -> run ============================================================== [1/3] Checking toolchain... OK: Visual Studio C:\Program Files (x86)\Microsoft Visual Studio\2022\BuildTools OK: CMake C:\Program Files\CMake\bin\cmake.exe OK: Ninja [2/3] Setting up vcpkg at C:\vcpkg... OK: vcpkg ready [3/3] Installing dependencies from vcpkg.json... [1/2] Configuring... [2/2] Building all targets... ============================================================== BUILD OK ==============================================================

Figure 7  —  Windows installation flow, with the two hazards marked

## Step 4.5  What was installed and where

| Path | Contents | Why here |
| --- | --- | --- |
| C:\vcpkg | vcpkg clone | Short path — see 4.6 |
| C:\vb | vcpkg build trees | Short path — see 4.6 |
| C:\vp | vcpkg packages | Short path — see 4.6 |
| C:\vi | installed headers, libs, DLLs | Short path — see 4.6 |
| cmake-build-msvc-static-release\ | build output and the executable | In the repository |

Figure 8  —  Where the installation puts things, and the runtime DLL requirement

## 4.6  Why the short paths are mandatory

vcpkg builds libiconv with autotools. During make install it forms a path by concatenating the package directory with the full install prefix. Under a deep project path the result exceeds the Windows 260-character limit, libtool falls back to the long-path form, and cl.exe rejects it as an unknown option:

cl : Command line warning D9002 : ignoring unknown option '//?/C:/.../iconv.lib' iconv.obj : error LNK2019: unresolved external symbol libiconv_open .libs\iconv.exe : fatal error LNK1120: 7 unresolved externals

The import library is silently dropped and the port fails after roughly 40 minutes of work. Do not repoint these roots at longer paths.

# 5  Installation — Linux

Linux uses system libraries rather than vcpkg, because the GCC presets carry no vcpkg toolchain file. libiconv is therefore never built from source and the failure in 4.6 cannot occur.

## Step 5.1  Obtain the source

git clone <repository-url> arinc-615a-tool-suite cd arinc-615a-tool-suite

## Step 5.2  Install, build and run — one command

./build.sh ./build.sh -c Find ./build.sh debug -c Find ./build.sh --no-run

sudo is invoked for package installation; you will be prompted for your password.

Figure 9  —  Linux installation flow: distro packages, no vcpkg

## Step 5.3  Packages installed per distribution

| Need | apt | dnf | pacman | zypper |
| --- | --- | --- | --- | --- |
| Compiler | build-essential g++ | gcc-c++ | base-devel | gcc-c++ |
| Generator | ninja-build | ninja-build | ninja | ninja |
| Boost | libboost-all-dev | boost-devel | boost | boost-devel |
| spdlog | libspdlog-dev | spdlog-devel | spdlog | spdlog-devel |
| fmt | libfmt-dev | fmt-devel | fmt | fmt-devel |
| libxml++ | libxml++5.0-dev / 2.6 | libxml++-devel | libxml++ | libxml++-devel |

If the distribution's CMake is older than 4.3 — most are — the installer downloads the official Kitware binary into a git-ignored .toolchain/ directory and uses that. Override the version with CMAKE_VERSION=4.4.2 scripts/install-deps.sh.

# 6  Post-installation verification

Three checks confirm a good installation before any operational use.

## Check 6.1  The executable exists

REM Windows dir cmake-build-msvc-static-release\app\arinc_615a_operation\arinc_615a_operation.exe # Linux ls -l cmake-build-gcc-static-release/app/arinc_615a_operation/arinc_615a_operation

## Check 6.2  Windows only — put the DLLs on PATH

The Windows build links against the dynamic x64-windows triplet with VCPKG_APPLOCAL_DEPS=OFF, so dependency DLLs are not copied next to the executable. build.bat handles this; running the exe directly does not. Match the configuration — never mix release DLLs into a debug binary.

set "PATH=C:\vi\x64-windows\bin;%PATH%" REM release set "PATH=C:\vi\x64-windows\debug\bin;%PATH%" REM debug

Linux needs nothing here — system libraries are already on the loader search path.

## Check 6.3  The tool responds

arinc_615a_operation.exe --help

# 7  Operating procedures

## 7.1  Command syntax

arinc_615a_operation.exe -c <Command> [options] arinc_615a_operation.exe -c <Command> --help arinc_615a_operation.exe --help

## 7.2  MANDATORY — option syntax

Always use the long option form with '='. For example --timeout=5, not -t 5.

Space-separated values are mis-consumed, and short flags are reused for different meanings across option groups, so a short flag can silently bind to the wrong option:

| Short flag | Means | But also means |
| --- | --- | --- |
| -l | --log-level | --targets-list |
| -i | --target-id | --timeout-option (TFTP) |
| -t | --timeout (Find) | --tftp-timeout |
| -p | --dynamic-port (Find) | --server-port (TFTP) |
| -d | --media-set-manager-dir | --dally (TFTP) |
| -c | --command | --check-file-integrity, --no-check-validity |

## 7.3  Addressing a target

REM directly by IP --target-address=10.0.0.5 --target-id=THW-1 REM or resolve the ID through a list produced by Find --targets-list=targets.json --target-id=THW-1

--target-id is required on every operation. BatchUpload accepts only --targets-list, because a batch spans multiple targets.

## 7.4  Options common to all ARINC 615A operations

| Option | Default | Purpose |
| --- | --- | --- |
| --dlp-timeout=<s> | 13 | Exception timer; fails a stalled operation |
| --dlp-retries=<n> | — | Protocol retry count |
| --port-option | off | Enable the ARINC 615A-3 Port Option |
| --local-tftp-address=<ip> | — | Interface the TFTP server binds to |
| --log-protocol-files | off | Log every protocol file sent and received |
| --log-level=<lvl> | warn | trace, debug, info, warn, err, critical, off |
| --server-port=<port> | — | Local TFTP server UDP port |
| --tftp-timeout=<s> | — | Packet timeout when none is negotiated |
| --dally | — | Wait after final ACK to survive a lost last ACK |
| --block-size-option[=n] | 1468 | Negotiate TFTP block size, 8 to 65464 |
| --timeout-option[=s] | 2 | Negotiate TFTP timeout, 1 to 255 |
| --handle-transfer-size-option | — | Negotiate the transfer size option |

## 7.5  Typical operational session

REM 1. discover what is on the network arinc_615a_operation.exe -c Find --timeout=5 --targets-list=targets.json REM 2. read what is currently loaded on a target arinc_615a_operation.exe -c Information --targets-list=targets.json --target-id=THW-1 REM 3. stage the software locally arinc_615a_operation.exe -c Create --media-set-manager-dir=C:\msm arinc_615a_operation.exe -c ImportMediaSet --media-set-manager-dir=C:\msm --source-directory=D:\medium1 arinc_615a_operation.exe -c ListLoads --media-set-manager-dir=C:\msm REM 4. upload arinc_615a_operation.exe -c Upload --targets-list=targets.json --target-id=THW-1 --media-set-manager-dir=C:\msm --media-set-pn=ABC12-3456-7890 --load-header=LOAD001.LUH REM 5. confirm the change took effect arinc_615a_operation.exe -c Information --targets-list=targets.json --target-id=THW-1

Figure 10  —  FIND discovery: one broadcast, N answers, bounded by --timeout

Figure 11  —  Information operation: LCI out, LCS status loop, LCL result

Figure 12  —  Upload: the host becomes a TFTP server and the target pulls the files

## 7.6  Stopping an operation

Press Ctrl-C once for a graceful protocol abort — the tool tells the target to stop. Press Ctrl-C a second time only if the first does not return; that terminates hard and may leave the target in an undefined state. On real hardware, allow the first abort to complete.

# 8  Test procedure

Execute in order. Tests 1 to 12 need no target hardware and form the acceptance set for an installation. Tests 13 to 16 require hardware or a Linux host.

| Legend | Meaning |
| --- | --- |
| VERIFIED | Executed on Windows x64; the expected output shown is the actual output |
| NOT EXECUTED | Specified from the tool's option definitions; not run |

Figure 13  —  Test sequence and the acceptance gate

## 8.1  Test environment used for verification

| Item | Value |
| --- | --- |
| OS | Windows 11 x64 |
| Compiler | MSVC 14.44 (Visual Studio 2022 Build Tools) |
| CMake / generator | 4.4.2 / Ninja |
| Configuration | msvc-static-release |
| Executable size | 2,091,520 bytes |
| Network | No ARINC 615A target hardware present |

# 9  Test cases

### TC-01  Toolchain prerequisites are detected

| Field | Detail |
| --- | --- |
| Objective | Confirm the installer finds the compiler, CMake 4.3+ and Ninja. |
| Preconditions | Visual Studio 2022 C++ tools and CMake 4.3+ installed. |
| Verification status | VERIFIED |

Procedure

scripts\install-deps.bat

Expected result

[1/3] Checking toolchain... OK: Visual Studio C:\Program Files (x86)\Microsoft Visual Studio\2022\BuildTools OK: CMake C:\Program Files\CMake\bin\cmake.exe OK: Ninja Exit code: 0

### TC-02  One-command build from a clean tree

| Field | Detail |
| --- | --- |
| Objective | Confirm dependencies install and all targets build. |
| Preconditions | TC-01 passed. Network access available. |
| Verification status | VERIFIED |

Procedure

build.bat --no-run

Expected result

==== BUILD OK ==== Executable: ...\cmake-build-msvc-static-release\app\arinc_615a_operation\arinc_615a_operation.exe Exit code: 0 Note: first run may take hours (libiconv). Later runs, seconds.

### TC-03  Build artefact is produced

| Field | Detail |
| --- | --- |
| Objective | Confirm the executable exists and is non-trivial in size. |
| Preconditions | TC-02 passed. |
| Verification status | VERIFIED |

Procedure

dir cmake-build-msvc-static-release\app\arinc_615a_operation\arinc_615a_operation.exe

Expected result

One file listed, size approximately 2,091,520 bytes (Release). Debug builds are larger, approximately 9 MB.

### TC-04  Command catalogue is complete

| Field | Detail |
| --- | --- |
| Objective | Confirm all 16 commands register. |
| Preconditions | TC-03 passed; DLLs on PATH per 6.2. |
| Verification status | VERIFIED |

Procedure

arinc_615a_operation.exe --help

Expected result

ARINC 615A Operation - (<git-hash>) Command Options: -c [ --command ] command Command to execute -h [ --help ] Help Commands are: * AdhocUpload * BatchUpload * Create * Find * ImportMediaSet * ImportMediaSetXml * Information * ListBatches * ListLoads * ListMediaSets * MedDownload * OpDownload * RemoveMediaSet * Targets * Upload * UploadLoads PASS if all 16 names appear. Exit code: 1 (help with no command)

### TC-05  Per-command help dispatches

| Field | Detail |
| --- | --- |
| Objective | Confirm a sub-command parses and reports its own options. |
| Preconditions | TC-04 passed. |
| Verification status | VERIFIED |

Procedure

arinc_615a_operation.exe -c ListMediaSets --help

Expected result

List all Media Sets registered with the Media Set Manager. List ARINC 665 Media Sets Options: -d [ --media-set-manager-dir ] Directory ARINC 665 Media Set Manager directory. Required. Exit code: 0

### TC-06  FIND discovery executes and terminates

| Field | Detail |
| --- | --- |
| Objective | Confirm the protocol stack opens a socket, broadcasts, honours its timeout and shuts down cleanly. This is the primary smoke test. |
| Preconditions | TC-04 passed. Any network interface up. |
| Verification status | VERIFIED |

Procedure

arinc_615a_operation.exe -c Find

Expected result

ARINC 615A Operation - (<git-hash>) ARINC 615A FIND Query ARINC 615A FIND Query finished Exit code: 0 With no target hardware present, zero targets are reported. That is a PASS — the test proves the stack runs and terminates, not that hardware exists.

### TC-07  Media Set Manager can be created

| Field | Detail |
| --- | --- |
| Objective | Confirm the ARINC 665 local store initialises. |
| Preconditions | TC-04 passed. |
| Verification status | VERIFIED |

Procedure

arinc_615a_operation.exe -c Create --media-set-manager-dir=%TEMP%\msm-test

Expected result

Create ARINC 665 Media Set Manager Media Set Manager directory: C:\Users\<user>\AppData\Local\Temp\msm-test Exit code: 0 A file MediaSetManager.json (6 bytes when empty) is created in that directory.

### TC-08  Empty store reports empty

| Field | Detail |
| --- | --- |
| Objective | Confirm the list commands read the store and report correctly when it is empty. |
| Preconditions | TC-07 passed. |
| Verification status | VERIFIED |

Procedure

arinc_615a_operation.exe -c ListMediaSets --media-set-manager-dir=%TEMP%\msm-test arinc_615a_operation.exe -c ListLoads --media-set-manager-dir=%TEMP%\msm-test arinc_615a_operation.exe -c ListBatches --media-set-manager-dir=%TEMP%\msm-test

Expected result

List ARINC 665 Media Sets *** No media sets within media set manger *** List ARINC 665 Loads *** No loads within media set manger *** List ARINC 665 Batches *** No batches within media set manger *** Exit code: 0 for all three. Note: 'manger' is the tool's own spelling; match it exactly when parsing output.

### TC-09  Negative — no command supplied

| Field | Detail |
| --- | --- |
| Objective | Confirm a missing command is rejected with guidance. |
| Preconditions | TC-04 passed. |
| Verification status | VERIFIED |

Procedure

arinc_615a_operation.exe

Expected result

Error parsing command line: the option 'command' is required but missing Enter <path>\arinc_615a_operation.exe --help for command line description. Exit code: 1

### TC-10  Negative — unknown command

| Field | Detail |
| --- | --- |
| Objective | Confirm an invalid command name is rejected. |
| Preconditions | TC-04 passed. |
| Verification status | VERIFIED |

Procedure

arinc_615a_operation.exe -c NoSuchCommand

Expected result

Error parsing command line: the argument ('NoSuchCommand') for option is invalid Enter <path> --command NoSuchCommand --help for command line description. Exit code: 1

### TC-11  Negative — required option omitted

| Field | Detail |
| --- | --- |
| Objective | Confirm required options are enforced per command. |
| Preconditions | TC-04 passed. |
| Verification status | VERIFIED |

Procedure

arinc_615a_operation.exe -c Information --target-address=127.0.0.1 arinc_615a_operation.exe -c ListMediaSets

Expected result

ARINC 615A Information Operation Error parsing command line: the option '--target-id' is required but missing List ARINC 665 Media Sets Error parsing command line: the option '--media-set-manager-dir' is required but missing Exit code: 1 for both

### TC-12  Release package is self-contained

| Field | Detail |
| --- | --- |
| Objective | Confirm the published binary runs with no vcpkg directory on PATH. |
| Preconditions | Release zip downloaded and extracted. |
| Verification status | VERIFIED |

Procedure

REM in a shell whose PATH contains only C:\Windows and C:\Windows\system32 arinc_615a_operation.exe -c Find

Expected result

ARINC 615A Operation - (<git-hash>) ARINC 615A FIND Query ARINC 615A FIND Query finished Exit code: 0 PASS requires no missing-DLL dialog. The package bundles six DLLs: boost_program_options, fmt, iconv-2, libxml2, xml++, z.

### TC-13  Information operation against a target

| Field | Detail |
| --- | --- |
| Objective | Read part numbers and versions from real target hardware. |
| Preconditions | An ARINC 615A target reachable on the network; its IP and target ID known. |
| Verification status | NOT EXECUTED — requires target hardware |

Procedure

arinc_615a_operation.exe -c Information --target-address=<ip> --target-id=<id> --log-level=info

Expected result

The host writes an LCI, receives LCS status files while the target works, and finally an LCL listing target hardware IDs and loaded part numbers. PASS: part numbers are printed and the exit code is 0. FAIL: operation aborted by DLP (no status within --dlp-timeout, default 13 s).

### TC-14  Upload operation

| Field | Detail |
| --- | --- |
| Objective | Upload a load from a registered media set to a target. |
| Preconditions | TC-13 passed. A media set imported and its P/N and load header known. |
| Verification status | NOT EXECUTED — requires target hardware |

Procedure

arinc_615a_operation.exe -c Upload --target-address=<ip> --target-id=<id> --media-set-manager-dir=<dir> --media-set-pn=<pn> --load-header=<file>

Expected result

Progress is reported from LUS status files until a terminating status. PASS: exit code 0, and a subsequent TC-13 reports the new part number. Verify the change actually took effect; do not rely on exit code alone.

### TC-15  Download operation

| Field | Detail |
| --- | --- |
| Objective | Retrieve files from a target. |
| Preconditions | TC-13 passed. |
| Verification status | NOT EXECUTED — requires target hardware |

Procedure

REM list what is offered without downloading — omit --file arinc_615a_operation.exe -c MedDownload --target-address=<ip> --target-id=<id> REM then fetch specific files arinc_615a_operation.exe -c MedDownload --target-address=<ip> --target-id=<id> --file=<name> --download-base-directory=<dir>

Expected result

Without --file the operation cancels after the list transfer, showing what is available. With --file, the named files are written to a sub-directory of --download-base-directory (default: current directory). PASS: requested files present on disk and validity check passes; exit code 0.

### TC-16  Linux build and run

| Field | Detail |
| --- | --- |
| Objective | Confirm the Linux path installs, builds and runs. |
| Preconditions | A supported Linux distribution with sudo and network access. |
| Verification status | NOT EXECUTED — no Linux environment was available |

Procedure

./build.sh --no-run cmake-build-gcc-static-release/app/arinc_615a_operation/arinc_615a_operation -c Find

Expected result

BUILD OK, then the same FIND output as TC-06, exit code 0. No PATH configuration is needed on Linux. The Linux scripts are syntax-checked only. Distro package names are the most likely thing to need adjusting.

## 9.1  Acceptance criteria

An installation is acceptable when TC-01 through TC-12 all pass. TC-13 through TC-15 form the operational acceptance set and can only be executed against target hardware. TC-16 applies to Linux hosts.

# 10  Troubleshooting

| Symptom | Cause | Action |
| --- | --- | --- |
| LNK2019 / LNK1120 building libiconv; 'ignoring unknown option //?/C:/...' | Path length. vcpkg concatenates the install prefix onto the package directory, exceeding 260 characters; libtool emits the long-path form and cl.exe drops the .lib. | Ensure vcpkg roots are C:\vcpkg, C:\vb, C:\vp, C:\vi. Do not relocate to longer paths. |
| error C2220 with warning C4127 inside a Boost header | The presets combine /WX with /external:templates-, so a warning in a dependency's headers becomes an error. | Use scripts\build.bat, which passes -DCMAKE_COMPILE_WARNING_AS_ERROR=OFF. |
| CMake Error: No target "arinc_615a_download_request_file" | InstallPackage.cmake referenced a target that app/CMakeLists.txt no longer builds; fails at generate time, after configure succeeds. | Already fixed here with if( TARGET ... ). Re-apply if merging upstream changes. |
| Configure fails cloning dependencies | No network access to git.thomas-vogt.de. There is no offline fallback. | Restore access, or place checkouts beside the source tree so CMake picks them up. |
| Application fails to start with a missing-DLL error | Dynamic triplet with VCPKG_APPLOCAL_DEPS=OFF; DLLs are not beside the executable. | Set PATH per 6.2, matching debug/release, or use the Release zip which bundles them. |
| CMake reports a version below 4.3 | The CMake bundled with Visual Studio is 3.31 and appears first on PATH after vcvars. | Install CMake 4.3+; the script searches for a qualifying installation. |
| 'the required argument for option --timeout is missing' | Space-separated short option. | Use --timeout=5, not -t 5. |
| 'the argument (--timeout) for option --log-level is invalid' | -l is bound to both --log-level and --targets-list in Find. | Avoid -l on that sub-command; use long forms with '='. |
| Operation aborted by DLP | The target sent no status file within the exception timer. | Check connectivity and target state; raise --dlp-timeout; add --log-protocol-files. |
| Build appears frozen for a long time | libiconv's autotools configure. vcpkg buffers a port's output until it finishes. | Normal. Watch file timestamps under C:\vb\libiconv rather than the console. |

# 11  Appendices

## Appendix A  Exit codes

| Code | Meaning |
| --- | --- |
| 0 | Success, including a sub-command --help |
| 1 | Argument error, protocol failure, or top-level --help with no command |

## Appendix B  Command index

| Command | Group | Needs target hardware |
| --- | --- | --- |
| Find | Discovery | No — safe smoke test |
| Targets | Discovery | No — reads a saved list |
| Information | 615A operation | Yes |
| Upload | 615A operation | Yes |
| AdhocUpload | 615A operation | Yes |
| BatchUpload | 615A operation | Yes |
| UploadLoads | 615A operation | Yes |
| MedDownload | 615A operation | Yes |
| OpDownload | 615A operation | Yes |
| Create | ARINC 665 store | No |
| ImportMediaSet | ARINC 665 store | No |
| ImportMediaSetXml | ARINC 665 store | No |
| ListMediaSets | ARINC 665 store | No |
| ListLoads | ARINC 665 store | No |
| ListBatches | ARINC 665 store | No |
| RemoveMediaSet | ARINC 665 store | No |

## Appendix C  Related documents

| Document | Contents |
| --- | --- |
| README.md | Overview, diagrams, quick start, command list |
| docs/BUILD.md | Build stages, dependencies, failure modes, linked resources |
| docs/ARCHITECTURE.md | Layer map, directory walkthrough, design conventions |
| docs/CODE-TRACE.md | Function-by-function trace from main() to the wire |

## Appendix D  Standards

| Reference | Title |
| --- | --- |
| ARINC 615A-4 | Software Data Loader Using Ethernet Interface |
| ARINC 665-5 | Loadable Software Standards |
| ARINC 649 | Common Terminology and Functions for Software Distribution and Loading |
| RFC 1350 | The TFTP Protocol (Revision 2) |
| RFC 2347/2348/2349 | TFTP Option Extension, Blocksize, Timeout and Transfer Size |

ARINC standards are published by SAE ITC and are not redistributed with this software. ARINC is a trademark of its respective owner; this project implements the publicly documented protocol and is not affiliated with or endorsed by ARINC.